Robot: Robomaster 24

### Python API:

In my experiments, I found this reference to be outdated, and many provided methods did not work on my end. The following are my notes on the data provided in the API:

- robotic_arm_ctrl.move(x,y,wait_for_complete=True)

  - I found increased accuracy when using the translation on one coordinate at a time: robotic_arm_ctrl.move(y=_) or (x=)

  - Wait for complete should not be a parameter, instead it is added at the end of the statement:

    - robotic_arm_ctrl.move(x=0).wait_for_completed()

- robotic_arm_ctrl.moveto(x,y, wait_for_complete=True)

  - This method worked inconsistently during testing, through the arm was translated, its ending coordinates were frequently very different from the ones passed in the method initially.

  - Wait for complete should not be a parameter, instead it is added at the end of the statement:

    - robotic_arm_ctrl.move(100,100).wait_for_completed()

- robotic_arm_ctrl.get_position()

  - This method does not work and is outdated. Here is the alternative way to get the arms position:

def sub_data_handler(sub_info):

x, y = sub_info

ep_arm.sub_position(freq=5, callback=sub_data_handler)

ep_arm.unsub_position() # At the end of program

- robotic_arm_ctrl.recenter()

  - This method recenters the arm, however depending on its initial position, its recentering could be jagged or not functional if it passes through an uncrossable zone.

### Arm control initialization process:

from robomaster import robot

if __name__ == ‘__main__’:

ep_robot = robot.Robot()

ep_robot.initialize(conn_type=”ap”)

ep_arm = ep_robot.robotic_arm

# arm control code, calling methods using ‘ep_arm’

ep_robot.close()

### Integer Overflow:

For some controls and movements, the x and y values dip below zero and results in an integer overflow error. This can cause issues when using coordinates to control the arm position.

Robomaster uses 32 bit integers to store the arm position. Range (0 - 4,294,967,296)